四轴无人机_遥控器

（作者：林耿超）

版本：V1.1.0

项目需求

本遥控器是为了对无人机实现远程操控而设计

系统总体设计

公共层

Common_Config.h

#ifndef __COMMON_CONFIG_H

#define __COMMON_CONFIG_H

/* 0. 定义通讯的帧头 */

#define FRAME_0 0xAA

#define FRAME_1 0xAF

#define FRAME_2 0x03

/* 1. 通用状态枚举类型 */

typedef enum

{

Common_OK = 0,

Common_ERROR,

Common_OTHER

} Common_Status;

/* 2. 遥感和按键数据封装结构体  */

typedef struct

{

uint16_t THR; /* 油门 Throttle */

uint16_t YAW; /* 横滚 */

uint16_t ROL; /* 横滚 */

uint16_t PIT; /* 俯仰 */

uint8_t isFixHeightPoint; /* 是否定高定点 */

uint8_t isPowerDonw;      /* 是否关机 */

} JoyStick_Struct;

typedef struct

{

int16_t THRBias; /* 油门 偏差 */

int16_t YAWBias; /* 横滚 偏差*/

int16_t ROLBias; /* 横滚 偏差*/

int16_t PITBias; /* 俯仰 偏差*/

} JoyStickBias_Struct;

#endif

电源模块

参考飞控板.

手柄和按键数据处理模块

STM32CubeMX配置

按键配置

手柄配置

硬件接口层

Inf_JoyStickKey.h

#ifndef __INF_JOYSTICKKEY_H

#define __INF_JOYSTICKKEY_H

#include "Common_Debug.h"

#include "Common_Config.h"

#include "Common_Delay.h"

#include "main.h"

#include "adc.h"

#define READ_KEY_LEFT (HAL_GPIO_ReadPin(KEY_LEFT_GPIO_Port, KEY_LEFT_Pin))

#define READ_KEY_RIGHT (HAL_GPIO_ReadPin(KEY_RIGHT_GPIO_Port, KEY_RIGHT_Pin))

#define READ_KEY_UP (HAL_GPIO_ReadPin(KEY_UP_GPIO_Port, KEY_UP_Pin))

#define READ_KEY_DOWN (HAL_GPIO_ReadPin(KEY_DOWN_GPIO_Port, KEY_DOWN_Pin))

#define READ_KEY_LEFT_UP (HAL_GPIO_ReadPin(KEY_LEFT_UP_GPIO_Port, KEY_LEFT_UP_Pin))

#define READ_KEY_RIGHT_UP (HAL_GPIO_ReadPin(KEY_RIGHT_UP_GPIO_Port, KEY_RIGHT_UP_Pin))

#define NOKEY_PRESS 0

#define KEY_LEFT_PRESS 1

#define KEY_RIGHT_PRESS 2

#define KEY_UP_PRESS 3

#define KEY_DOWN_PRESS 4

#define KEY_LEFT_UP_PRESS 5

#define KEY_RIGHT_UP_PRESS 6

#define KEY_LEFT_UP_LONG_PRESS 7

#define KEY_RIGHT_UP_LONG_PRESS 8

extern JoyStick_Struct joyStick;

void    Inf_JoyStickKey_Init(void);

void    Inf_JoyStickKey_JoyStickScan(void);

uint8_t Inf_JoyStickKey_KeyScan(void);

#endif

Inf_JoyStickKey.c

#include "Inf_JoyStickKey.h"

/* 存储摇杆数据:

0: left杆->   左右 -> 偏航

1: left杆->   前后 -> 油门

2: right杆->  左右 -> 横滚

3: right杆->  前后 -> 俯仰

*/

uint16_t        joyStickBuff[4];

JoyStick_Struct joyStick;

/**

* @description: 摇杆和按键初始化

* @return {*}

*/

void Inf_JoyStickKey_Init(void)

{

/* 启动ADC采集, 因为有4个通道, 所以要开启DMA */

debug_printfln("摇杆和按键模块: 开始启动");

HAL_ADCEx_Calibration_Start(&hadc1);

HAL_ADC_Start_DMA(&hadc1, (uint32_t *)joyStickBuff, 4);

debug_printfln("摇杆和按键模块: 启动完成");

}

/**

* @description: 扫描遥感数据

* @return {*}

*/

void Inf_JoyStickKey_JoyStickScan(void)

{

joyStick.THR = joyStickBuff[0];

joyStick.YAW = joyStickBuff[1];

joyStick.PIT = joyStickBuff[2];

joyStick.ROL = joyStickBuff[3];

}

/**

* @description: 按键扫描. 检测按键是否被按下

* @return {*}

*/

uint8_t Inf_JoyStickKey_KeyScan(void)

{

/* 表示有按键按下 */

if(READ_KEY_LEFT == 0 || READ_KEY_RIGHT == 0 ||

READ_KEY_UP == 0 || READ_KEY_DOWN == 0 ||

READ_KEY_LEFT_UP == 0 || READ_KEY_RIGHT_UP == 0)

{

common_delay_ms(50);

if(READ_KEY_LEFT == 0)

{

/* 等待按键松开 */

while(READ_KEY_LEFT == 0);

return KEY_LEFT_PRESS;

}

else if(READ_KEY_RIGHT == 0)

{

while(READ_KEY_RIGHT == 0);

return KEY_RIGHT_PRESS;

}

else if(READ_KEY_UP == 0)

{

while(READ_KEY_UP == 0);

return KEY_UP_PRESS;

}

else if(READ_KEY_DOWN == 0)

{

while(READ_KEY_DOWN == 0);

return KEY_DOWN_PRESS;

}

else if(READ_KEY_LEFT_UP == 0)

{

uint16_t time = 0;

while(READ_KEY_LEFT_UP == 0 && time < 1200)

{

common_delay_ms(1);

time++;

}

while(READ_KEY_LEFT_UP == 0); /* 等待松开 */

if(time < 500)                /* 松开之后判断是长按or短按 */

{

return KEY_LEFT_UP_PRESS;

}

else

{

return KEY_LEFT_UP_PRESS;

}

}

else if(READ_KEY_RIGHT_UP == 0)

{

uint16_t time = 0;

/* 等待按键松开 */

while(READ_KEY_RIGHT_UP == 0 && time < 1200) /* 最多等待 1.2s */

{

common_delay_ms(1);

time++;

}

while(READ_KEY_RIGHT_UP == 0); /* 等待松开 */

if(time < 500) /* 在500ms松开, 证明是一次普通的点按 */

{

return KEY_RIGHT_UP_PRESS;

}

else /* 超过500ms证明是长按 */

{

return KEY_RIGHT_UP_LONG_PRESS;

}

}

}

return NOKEY_PRESS;

}

应用层

App_DataProcess.h

#ifndef __APP_DATAPROCESS_H

#define __APP_DATAPROCESS_H

#include "Common_Debug.h"

#include "Inf_JoyStickKey.h"

/* 对x限幅度处理 */

#define LIMIT(x, min, max) ((x) < (min) ? (min) : ((x) > (max) ? (max) : (x)))

/* 均值滤波缓冲区大小 */

#define FILTER_SIZE 10

/* 滤波缓冲区结构体 */

typedef struct

{

int32_t values[10]; /* 存储最近的10个值 */

int32_t index;      /* 索引 */

int32_t sum;        /* 存储10个值的和 */

} FilterBuffer_Struct;

void App_DataProcess_Start(void);

void App_DataProcess_JoyStickDataProcess(void);

void App_DataProcess_KeyDataProcess(void);

#endif

App_DataProcess.c

#include "App_DataProcess.h"

/* 均值滤波(滑动窗口) */

#define MEAN_FILTER(filter, valuePointer)             \

filter.sum -= filter.values[filter.index];        \

filter.values[filter.index] = *valuePointer;      \

filter.sum += filter.values[filter.index];        \

filter.index  = (filter.index + 1) % FILTER_SIZE; \

*valuePointer = filter.sum / FILTER_SIZE

JoyStickBias_Struct JoyStickBias;

/**

* @description: 摇杆和按键数据处理模块启动

* @return {*}

*/

void App_DataProcess_Start(void)

{

debug_printfln("摇杆和按键数据处理模块开始启动");

Inf_JoyStickKey_Init();

Inf_JoyStickKey_JoyStickScan();

debug_printfln("摇杆和按键数据处理模块启动完成");

}

/**

* @description: 摇杆极性数据处理

* @return {*}

*/

static void App_DataProcess_JoyStickDataPolarity(void)

{

/* 摇杆的极性, 与我们的感知正好相反, 先修改极性 */

joyStick.THR = 1000 - joyStick.THR * 1000 / 4095;

joyStick.YAW = 1000 - joyStick.YAW * 1000 / 4095;

joyStick.PIT = 1000 - joyStick.PIT * 1000 / 4095;

joyStick.ROL = 1000 - joyStick.ROL * 1000 / 4095;

}

/**

* @description: 考虑摇杆的读取的值的偏移

* @return {*}

*/

static void App_DataProcess_JoyStickDataWithBias(void)

{

/* 为了防止出现负数, 对移除偏移量的摇杆数据进行限幅处理 */

joyStick.THR = LIMIT(joyStick.THR - JoyStickBias.THRBias, 0, 1000);

joyStick.YAW = LIMIT(joyStick.YAW - JoyStickBias.YAWBias, 0, 1000);

joyStick.PIT = LIMIT(joyStick.PIT - JoyStickBias.PITBias, 0, 1000);

joyStick.ROL = LIMIT(joyStick.ROL - JoyStickBias.ROLBias, 0, 1000);

}

/* 初始化滤波缓冲区 */

static FilterBuffer_Struct THRFilter = {{0}, 0, 0};

static FilterBuffer_Struct YAWFilter = {{0}, 0, 0};

static FilterBuffer_Struct PITFilter = {{0}, 0, 0};

static FilterBuffer_Struct ROLFilter = {{0}, 0, 0};

/**

* @description: 对摇杆4个方向数据进行滤波处理

*              这次我们使用平均值滤波(滑动窗口滤波)

*              取最近的10个值的平均值

* @return {*}

*/

static void App_DataProcess_JoyStickDataFilter(void)

{

/*

// 1. 减去旧值

THRFilter.sum -= THRFilter.values[THRFilter.index];

// 2. 旧值的地方存储新值

THRFilter.values[THRFilter.index] = joyStick.THR;

// 3. 加上新值

THRFilter.sum += THRFilter.values[THRFilter.index];

// 4. 更新索引

THRFilter.index = (THRFilter.index + 1) % FILTER_SIZE;

//5. 计算均值

joyStick.THR = THRFilter.sum / FILTER_SIZE;

为了提高可读性,使用宏定义滤波算法

*/

MEAN_FILTER(THRFilter, &joyStick.THR);

MEAN_FILTER(YAWFilter, &joyStick.YAW);

MEAN_FILTER(PITFilter, &joyStick.PIT);

MEAN_FILTER(ROLFilter, &joyStick.ROL);

}

/**

* @description: 处理摇杆数据

* @return {*}

*/

void App_DataProcess_JoyStickDataProcess(void)

{

Inf_JoyStickKey_JoyStickScan();         /* 扫描摇杆数据 */

App_DataProcess_JoyStickDataPolarity(); /* 对摇杆数据进行极性处理 */

App_DataProcess_JoyStickDataWithBias(); /* 考虑摇杆的偏差 */

App_DataProcess_JoyStickDataFilter(); /* 对摇杆数据进行滤波 */

}

/**

* @description: 对摇杆进行偏移量校准.  读取100次数据, 求出平均偏移量

*  当长按右上按键的时候,调用次函数进行校准

* @return {*}

*/

static void App_DataProcess_JoyStickCalcBias()

{

JoyStickBias.PITBias = 0;

JoyStickBias.ROLBias = 0;

JoyStickBias.YAWBias = 0;

JoyStickBias.ROLBias = 0;

/* 连续读取100次, 求取平均偏移量 */

for(uint8_t i = 0; i < 100; i++)

{

Inf_JoyStickKey_JoyStickScan();         /* 扫描摇杆数据 */

App_DataProcess_JoyStickDataPolarity(); /* 对摇杆数据进行极性处理 */

JoyStickBias.THRBias += joyStick.THR - 0;

JoyStickBias.YAWBias += joyStick.YAW - 500;

JoyStickBias.PITBias += joyStick.PIT - 500;

JoyStickBias.ROLBias += joyStick.ROL - 500;

}

JoyStickBias.THRBias /= 100;

JoyStickBias.YAWBias /= 100;

JoyStickBias.PITBias /= 100;

JoyStickBias.ROLBias /= 100;

}

/**

* @description: 按键数据处理:

* @return {*}

*/

void App_DataProcess_KeyDataProcess(void)

{

/*

1. 如果是右上按键长按, 则进行校准

THR 拉到最低, 实现零值校准  0

其他由于会自动回中, 则进行中值校准  500

4. 右上按键按一下启动定高定点

5. 左上按键按一下远程关闭飞控

*/

uint8_t key = Inf_JoyStickKey_KeyScan();

if(key == KEY_RIGHT_UP_LONG_PRESS) /* 右上按键长安: 实现零值校准 */

{

debug_printfln("长");

App_DataProcess_JoyStickCalcBias();

}

else if(key == KEY_LEFT_PRESS)

{

JoyStickBias.ROLBias += 10;

}

else if(key == KEY_UP_PRESS)

{

JoyStickBias.PITBias -= 10;

}

else if(key == KEY_RIGHT_PRESS)

{

JoyStickBias.ROLBias -= 10;

}

else if(key == KEY_DOWN_PRESS)

{

JoyStickBias.PITBias += 10;

}

else if(key == KEY_LEFT_UP_PRESS) /* 左上按键按下: 远程关键 */

{

joyStick.isPowerDonw = 1;

}

else if(key == KEY_RIGHT_UP_PRESS) /* 右上按键按下: 关闭和开启定高定点模式 */

{

debug_printfln("短");

joyStick.isFixHeightPoint = 1;

}

}

任务调度

/* 3. 摇杆和按键 数据处理 */

void         dataProessTask(void *arg);

TaskHandle_t dataProessHandle;

#define JOYSTICK_PRIORITY 8

#define JOYSTICK_EXECUTE_CYCLE 4

/* 3. 摇杆和按键数据处理 */

void dataProessTask(void *arg)

{

vTaskDelay(1000);

debug_printfln("摇杆和按键处理任务: 开始调度");

TickType_t pxPreviousWakeTime = xTaskGetTickCount();

while(1)

{

/* 1. 摇杆数据处理 */

App_DataProcess_JoyStickDataProcess();

/* 2. 按键处理 */

App_DataProcess_KeyDataProcess();

xTaskDelayUntil(&pxPreviousWakeTime, JOYSTICK_EXECUTE_CYCLE);

}

}

通讯模块

STM32CubeMX配置

参考飞控板配置

硬件接口层

参考飞控板

应用层

App_Communication.h

#ifndef __APP_COMMUNICATION_H

#define __APP_COMMUNICATION_H

#include "Common_Debug.h"

#include "Inf_Si24R1.h"

#include "Common_Delay.h"

#include "Common_Config.h"

void App_Communication_Start(void);

void App_Communication_SendJoyStickData(JoyStick_Struct *joyStick);

#endif

App_Communication.c

#include "App_Communication.h"

/**

* @description: 启动通讯模块

* @return {*}

*/

void App_Communication_Start(void)

{

debug_printfln("通讯模块开始启动");

/* 1. 等待自检通过 */

debug_printfln("    2.4G模块开始自检");

HAL_Delay(1000);

while(Inf_Si24R1_Check() == 1);

debug_printfln("    2.4G模块自检通过");

/* 2. 把2.4G模块设置为发送模式 */

debug_printfln("    2.4G模块设置为发送模式");

Inf_Si24R1_TXMode();

debug_printfln("通讯模块完成启动");

}

/**

* @description: 发送摇杆数据

* 协议: 4字节帧头+9字节有效载荷(摇杆+定高)+4字节校验和

* @return {*}

*/

void App_Communication_SendJoyStickData(JoyStick_Struct *joyStick)

{

/* 填充要发送的数据 */

uint8_t index = 0;

TX_BUFF[index++] = FRAME_0; /* 0-3 为帧头 */

TX_BUFF[index++] = FRAME_1;

TX_BUFF[index++] = FRAME_2;

TX_BUFF[index++] = 0x00; /* 存储有效数据字节数 */

TX_BUFF[index++] = (joyStick->THR >> 8);

TX_BUFF[index++] = (joyStick->THR & 0xff);

TX_BUFF[index++] = (joyStick->YAW >> 8);

TX_BUFF[index++] = (joyStick->YAW & 0xff);

TX_BUFF[index++] = (joyStick->ROL >> 8);

TX_BUFF[index++] = (joyStick->ROL & 0xff);

TX_BUFF[index++] = (joyStick->PIT >> 8);

TX_BUFF[index++] = (joyStick->PIT & 0xff);

TX_BUFF[index++] = joyStick->isFixHeightPoint; /* 定高定点命令0:不定高定点 1:定高定点 */

TX_BUFF[index++] = joyStick->isPowerDonw;      /* 关机命令0:不关机 1:关机 */

/* 关机命令只发一次即可. 发完之后,把关机命令重新清零 */

joyStick->isPowerDonw = 0;

joyStick->isFixHeightPoint = 0;

TX_BUFF[3] = index - 4; /* 有效载荷数 */

/* 存储 校验和, 接收收到数据可用用同样的方式进行校验*/

uint32_t checkSum = 0;

for(uint8_t i = 0; i < index; i++)

{

checkSum += TX_BUFF[i];

}

TX_BUFF[index++] = (checkSum >> 24);

TX_BUFF[index++] = (checkSum >> 16);

TX_BUFF[index++] = (checkSum >> 8);

TX_BUFF[index++] = checkSum;

uint8_t r = Inf_Si24R1_TxPacket(TX_BUFF);

if(r == 0)

{

// debug_printfln("发送成功 %d", checkSum);

}

else

{

// debug_printfln("发送失败");

}

}

任务调度

/* 2. 通讯任务 */

void         communicationTask(void *arg);

TaskHandle_t communicationHandle;

#define COMMUNICATION_PRIORITY 8

#define COMMUNICATION_EXECUTE_CYCLE 4

/* 2. 通讯任务执行函数 */

void communicationTask(void *arg)

{

vTaskDelay(1000);

debug_printfln("通讯任务: 开始调度");

TickType_t pxPreviousWakeTime = xTaskGetTickCount();

while(1)

{

taskENTER_CRITICAL();

App_Communication_SendJoyStickData(&joyStick);

taskEXIT_CRITICAL();

xTaskDelayUntil(&pxPreviousWakeTime, COMMUNICATION_EXECUTE_CYCLE);

}

}

显示模块

硬件接口层

oled.h

#ifndef __OLED_H

#define __OLED_H

#include "main.h"

#include "gpio.h"

#define OLED_CS_GPIO GPIOA

#define OLED_CS_Pin GPIO_PIN_4

#define OLED_DC_GPIO GPIOB

#define OLED_DC_Pin GPIO_PIN_1

#define OLED_RST_GPIO GPIOB

#define OLED_RST_Pin GPIO_PIN_0

#define OLED_SDIN_GPIO GPIOA

#define OLED_SDIN_Pin GPIO_PIN_7

#define OLED_SCLK_GPIO GPIOA

#define OLED_SCLK_Pin GPIO_PIN_5

//-----------------OLED端口定义----------------

#define OLED_CS_Clr() HAL_GPIO_WritePin(GPIOA,GPIO_PIN_4,GPIO_PIN_RESET) // CS

#define OLED_CS_Set() HAL_GPIO_WritePin(GPIOA,GPIO_PIN_4,GPIO_PIN_SET)   // CS

#define OLED_DC_Clr() HAL_GPIO_WritePin(GPIOB,GPIO_PIN_1,GPIO_PIN_RESET) // DC

#define OLED_DC_Set() HAL_GPIO_WritePin(GPIOB,GPIO_PIN_1,GPIO_PIN_SET)   // DC

#define OLED_RST_Clr() HAL_GPIO_WritePin(GPIOB,GPIO_PIN_0,GPIO_PIN_RESET) // RST

#define OLED_RST_Set() HAL_GPIO_WritePin(GPIOB,GPIO_PIN_0,GPIO_PIN_SET)   // RST

#define OLED_SDIN_Clr() HAL_GPIO_WritePin(GPIOA,GPIO_PIN_7,GPIO_PIN_RESET) // SDA

#define OLED_SDIN_Set() HAL_GPIO_WritePin(GPIOA,GPIO_PIN_7,GPIO_PIN_SET)   // SDA

#define OLED_SCLK_Clr() HAL_GPIO_WritePin(GPIOA,GPIO_PIN_5,GPIO_PIN_RESET) // SCL

#define OLED_SCLK_Set() HAL_GPIO_WritePin(GPIOA,GPIO_PIN_5,GPIO_PIN_SET)   // SCL

#define OLED_CMD 0  // 写命令

#define OLED_DATA 1 // 写数据

// OLED控制用函数

void OLED_WR_Byte(uint8_t dat, uint8_t cmd);

void OLED_Display_On(void);

void OLED_Display_Off(void);

void OLED_Refresh_Gram(void);

void OLED_Init(void);

void OLED_Clear(void);

void OLED_DrawPoint(uint8_t x, uint8_t y, uint8_t t);

void OLED_ShowChar(uint8_t x, uint8_t y, uint8_t chr, uint8_t size, uint8_t mode);

void OLED_ShowNumber(uint8_t x, uint8_t y, uint32_t num, uint8_t len, uint8_t size);

void OLED_ShowString(uint8_t x, uint8_t y, const uint8_t *p, uint8_t size, uint8_t mode);

void OLED_ShowCH(uint8_t x, uint8_t y, uint8_t chr, uint8_t size, uint8_t mode);

void OLED_Show_CH(uint8_t x, uint8_t y, uint8_t chr, uint8_t size, uint8_t mode);

void OLED_Show_CH_String(uint8_t x, uint8_t y, const uint8_t *p, uint8_t size, uint8_t mode);

#endif

oled.c

#include "oled.h"

#include "stdlib.h"

#include "oledfont.h"

#include "gpio.h"

uint8_t OLED_GRAM[128][8];

void    OLED_Refresh_Gram(void)

{

uint8_t i, n;

for(i = 0; i < 8; i++)

{

OLED_WR_Byte(0xb0 + i, OLED_CMD);   // 设置页地址（0~7）

OLED_WR_Byte(0x00, OLED_CMD);       // 设置显示位置—列低地址

OLED_WR_Byte(0x10, OLED_CMD);       // 设置显示位置—列高地址

for(n = 0; n < 128; n++)

OLED_WR_Byte(OLED_GRAM[n][i], OLED_DATA);

}

}

// 向OLED写入一个字节。

// dat:要写入的数据/命令

// cmd:数据/命令标志 0,表示命令;1,表示数据;

void OLED_WR_Byte(uint8_t dat, uint8_t cmd)

{

uint8_t i;

if(cmd)

OLED_DC_Set();

else

OLED_DC_Clr();

OLED_CS_Clr(); /* 要加：先选中 */

for(i = 0; i < 8; i++)

{

OLED_SCLK_Clr();

if(dat & 0x80)

OLED_SDIN_Set();

else

OLED_SDIN_Clr();

OLED_SCLK_Set();

dat <<= 1;

}

OLED_CS_Set(); /* 要加：先释放 */

OLED_DC_Set();

}

// 开启OLED显示

void OLED_Display_On(void)

{

OLED_WR_Byte(0X8D, OLED_CMD);   // SET DCDC命令

OLED_WR_Byte(0X14, OLED_CMD);   // DCDC ON

OLED_WR_Byte(0XAF, OLED_CMD);   // DISPLAY ON

}

// 关闭OLED显示

void OLED_Display_Off(void)

{

OLED_WR_Byte(0X8D, OLED_CMD);   // SET DCDC命令

OLED_WR_Byte(0X10, OLED_CMD);   // DCDC OFF

OLED_WR_Byte(0XAE, OLED_CMD);   // DISPLAY OFF

}

// 清屏函数,清完屏,整个屏幕是黑色的!和没点亮一样!!!

void OLED_Clear(void)

{

uint8_t i, n;

for(i = 0; i < 8; i++)

{

for(n = 0; n < 128; n++)

{

OLED_GRAM[n][i] = 0X00;

}

}

OLED_Refresh_Gram();   // 更新显示

}

// 画点

// x:0~127

// y:0~63

// t:1 填充 0,清空

void OLED_DrawPoint(uint8_t x, uint8_t y, uint8_t t)

{

uint8_t pos, bx, temp = 0;

if(x > 127 || y > 63)

return;   // 超出范围了.

pos  = 7 - y / 8;

bx   = y % 8;

temp = 1 << (7 - bx);

if(t)

OLED_GRAM[x][pos] |= temp;

else

OLED_GRAM[x][pos] &= ~temp;

}

// 在指定位置显示一个字符,包括部分字符

// x:0~127

// y:0~63

// mode:0,反白显示;1,正常显示

// size:选择字体 16/12

void OLED_ShowChar(uint8_t x, uint8_t y, uint8_t chr, uint8_t size, uint8_t mode)

{

uint8_t temp, t, t1;

uint8_t y0 = y;

chr        = chr - ' ';   // 得到偏移后的值

for(t = 0; t < size; t++)

{

if(size == 12)

temp = oled_asc2_1206[chr][t];   // 调用1206字体

else

temp = oled_asc2_1608[chr][t];   // 调用1608字体

for(t1 = 0; t1 < 8; t1++)

{

if(temp & 0x80)

OLED_DrawPoint(x, y, mode);

else

OLED_DrawPoint(x, y, !mode);

temp <<= 1;

y++;

if((y - y0) == size)

{

y = y0;

x++;

break;

}

}

}

}

// 显示汉字

void OLED_ShowCH(uint8_t x, uint8_t y, uint8_t chr, uint8_t size, uint8_t mode)

{

uint8_t temp, t, t1;

uint8_t y0 = y;

for(t = 0; t < size; t++)

{

temp = oled_CH_1616[chr][t];   // 调用中文字库

for(t1 = 0; t1 < 8; t1++)

{

if(temp & 0x80)

OLED_DrawPoint(x, y, mode);

else

OLED_DrawPoint(x, y, !mode);

temp <<= 1;

y++;

if((y - y0) == size)

{

y = y0;

x++;

break;

}

}

}

}

// 显示单个中文字符

void OLED_Show_CH(uint8_t x, uint8_t y, uint8_t chr, uint8_t size, uint8_t mode)

{

OLED_ShowCH(x, y, chr * 2, size, 1);

OLED_ShowCH(x + size / 2, y, chr * 2 + 1, size, 1);

}

// 显示一串中文字符

void OLED_Show_CH_String(uint8_t x, uint8_t y, const uint8_t *p, uint8_t size, uint8_t mode)

{

uint8_t temp, t, t1;

uint8_t y0 = y;

for(t = 0; t < size; t++)

{

temp = p[t];   // 调用中文字库

for(t1 = 0; t1 < 8; t1++)

{

if(temp & 0x80)

OLED_DrawPoint(x, y, mode);

else

OLED_DrawPoint(x, y, !mode);

temp <<= 1;

y++;

if((y - y0) == size)

{

y = y0;

x++;

break;

}

}

}

}

// m^n函数

uint32_t oled_pow(uint8_t m, uint8_t n)

{

uint32_t result = 1;

while(n--)

result *= m;

return result;

}

// 显示2个数字

// x,y :起点坐标

// len :数字的位数

// size:字体大小

// mode:模式  0,填充模式;1,叠加模式

// num:数值(0~4294967295);

void OLED_ShowNumber(uint8_t x, uint8_t y, uint32_t num, uint8_t len, uint8_t size)

{

uint8_t t, temp;

uint8_t enshow = 0;

for(t = 0; t < len; t++)

{

temp = (num / oled_pow(10, len - t - 1)) % 10;

if(enshow == 0 && t < (len - 1))

{

if(temp == 0)

{

OLED_ShowChar(x + (size / 2) * t, y, ' ', size, 1);

continue;

}

else

enshow = 1;

}

OLED_ShowChar(x + (size / 2) * t, y, temp + '0', size, 1);

}

}

// 显示字符串

// x,y:起点坐标

//*p:字符串起始地址

// 用16字体

void OLED_ShowString(uint8_t x, uint8_t y, const uint8_t *p, uint8_t size, uint8_t mode)

{

#define MAX_CHAR_POSX 122

#define MAX_CHAR_POSY 58

while(*p != '\0')

{

if(x > MAX_CHAR_POSX)

{

x = 0;

y += 16;

}   // 换行

if(y > MAX_CHAR_POSY)

{

y = x = 0;

// OLED_Clear();

}

OLED_ShowChar(x, y, *p, size, mode);

x += 8;

p++;

}

}

// 显示字符串

// x,y:起点坐标

//*p:字符串起始地址

// 用16字体

void OLED_ShowString_16(uint8_t x, uint8_t y, const uint8_t *p)

{

#define MAX_CHAR_POSX 122

#define MAX_CHAR_POSY 58

while(*p != '\0')

{

if(x > MAX_CHAR_POSX)

{

x = 0;

y += 16;

}   // 换行

if(y > MAX_CHAR_POSY)

{

y = x = 0;

OLED_Clear();

}

OLED_ShowChar(x, y, *p, 16, 1);

x += 8;

p++;

}

}

// 初始化OLED

void OLED_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4 | GPIO_PIN_7, GPIO_PIN_SET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0 | GPIO_PIN_1, GPIO_PIN_SET);

GPIO_InitStruct.Pin   = GPIO_PIN_4 | GPIO_PIN_5 | GPIO_PIN_7;

GPIO_InitStruct.Mode  = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull  = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_HIGH;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

GPIO_InitStruct.Pin   = GPIO_PIN_0 | GPIO_PIN_1;

GPIO_InitStruct.Mode  = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull  = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_HIGH;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

OLED_RST_Clr();

HAL_Delay(100);

OLED_RST_Set();

OLED_WR_Byte(0xAE, OLED_CMD);   //--turn off oled panel

OLED_WR_Byte(0x00, OLED_CMD);   //---set low column address

OLED_WR_Byte(0x10, OLED_CMD);   //---set high column address

OLED_WR_Byte(0x40, OLED_CMD);   //--set start line address  Set Mapping RAM Display Start Line (0x00~0x3F)

OLED_WR_Byte(0x81, OLED_CMD);   //--set contrast control register

OLED_WR_Byte(0xCF, OLED_CMD);   // Set SEG Output Current Brightness

OLED_WR_Byte(0xA1, OLED_CMD);   //--Set SEG/Column Mapping     0xa0左右反置 0xa1正常

OLED_WR_Byte(0xC0, OLED_CMD);   // Set COM/Row Scan Direction   0xc0上下反置 0xc8正常

OLED_WR_Byte(0xA6, OLED_CMD);   //--set normal display

OLED_WR_Byte(0xA8, OLED_CMD);   //--set multiplex ratio(1 to 64)

OLED_WR_Byte(0x3f, OLED_CMD);   //--1/64 duty

OLED_WR_Byte(0xD3, OLED_CMD);   //-set display offset   Shift Mapping RAM Counter (0x00~0x3F)

OLED_WR_Byte(0x00, OLED_CMD);   //-not offset

OLED_WR_Byte(0xd5, OLED_CMD);   //--set display clock divide ratio/oscillator frequency

OLED_WR_Byte(0x80, OLED_CMD);   //--set divide ratio, Set Clock as 100 Frames/Sec

OLED_WR_Byte(0xD9, OLED_CMD);   //--set pre-charge period

OLED_WR_Byte(0xF1, OLED_CMD);   // Set Pre-Charge as 15 Clocks & Discharge as 1 Clock

OLED_WR_Byte(0xDA, OLED_CMD);   //--set com pins hardware configuration

OLED_WR_Byte(0x12, OLED_CMD);

OLED_WR_Byte(0xDB, OLED_CMD);   //--set vcomh

OLED_WR_Byte(0x30, OLED_CMD);   // Set VCOM Deselect Level

OLED_WR_Byte(0x20, OLED_CMD);   //-Set Page Addressing Mode (0x00/0x01/0x02)

OLED_WR_Byte(0x02, OLED_CMD);   //

OLED_WR_Byte(0x8D, OLED_CMD);   //--set Charge Pump enable/disable

OLED_WR_Byte(0x14, OLED_CMD);   //--set(0x10) disable

OLED_Clear();

OLED_WR_Byte(0xAF, OLED_CMD);

}

oledfont.h

#ifndef __OLEDFONT_H

#define __OLEDFONT_H

extern const unsigned char oled_asc2_1206[95][12];

extern const unsigned char oled_asc2_1608[95][16];

extern const unsigned char oled_CH_1616[][12];

extern const unsigned char oled_CH0[][12];

extern const unsigned char oled_CH1[][12];

extern const unsigned char oled_CH2[][12];

extern const unsigned char oled_CH3[][12];

extern const unsigned char oled_CH4[][12];

extern const unsigned char oled_CH5[][12];

extern const unsigned char oled_CH6[][12];

extern const unsigned char oled_CH7[][12];

extern const unsigned char oled_CH8[][12];

extern const unsigned char oled_CH9[][12];

extern const unsigned char oled_CH10[][12];

extern const unsigned char oled_CH11[][12];

extern const unsigned char oled_CH12[][12];

extern const unsigned char oled_CH13[][12];

extern const unsigned char oled_CH14[][12];

extern const unsigned char oled_CH15[][12];

extern const unsigned char oled_CH16[][12];

extern const unsigned char oled_CH17[][12];

extern const unsigned char oled_CH18[][12];

extern const unsigned char oled_CH19[][12];

extern const unsigned char oled_CH20[][12];

#endif

oledfont.c

#include "oledfont.h"

//常用ASCII表

//偏移量32

//ASCII字符集

//偏移量32

//大小:12*6

const unsigned char oled_asc2_1206[95][12]={

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*" ",0*/

{0x00,0x00,0x00,0x00,0x3F,0x40,0x00,0x00,0x00,0x00,0x00,0x00},/*"!",1*/

{0x00,0x00,0x30,0x00,0x40,0x00,0x30,0x00,0x40,0x00,0x00,0x00},/*""",2*/

{0x09,0x00,0x0B,0xC0,0x3D,0x00,0x0B,0xC0,0x3D,0x00,0x09,0x00},/*"#",3*/

{0x18,0xC0,0x24,0x40,0x7F,0xE0,0x22,0x40,0x31,0x80,0x00,0x00},/*"$",4*/

{0x18,0x00,0x24,0xC0,0x1B,0x00,0x0D,0x80,0x32,0x40,0x01,0x80},/*"%",5*/

{0x03,0x80,0x1C,0x40,0x27,0x40,0x1C,0x80,0x07,0x40,0x00,0x40},/*"&",6*/

{0x10,0x00,0x60,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"'",7*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x1F,0x80,0x20,0x40,0x40,0x20},/*"(",8*/

{0x00,0x00,0x40,0x20,0x20,0x40,0x1F,0x80,0x00,0x00,0x00,0x00},/*")",9*/

{0x09,0x00,0x06,0x00,0x1F,0x80,0x06,0x00,0x09,0x00,0x00,0x00},/*"*",10*/

{0x04,0x00,0x04,0x00,0x3F,0x80,0x04,0x00,0x04,0x00,0x00,0x00},/*"+",11*/

{0x00,0x10,0x00,0x60,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*",",12*/

{0x04,0x00,0x04,0x00,0x04,0x00,0x04,0x00,0x04,0x00,0x00,0x00},/*"-",13*/

{0x00,0x00,0x00,0x40,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*".",14*/

{0x00,0x20,0x01,0xC0,0x06,0x00,0x38,0x00,0x40,0x00,0x00,0x00},/*"/",15*/

{0x1F,0x80,0x20,0x40,0x20,0x40,0x20,0x40,0x1F,0x80,0x00,0x00},/*"0",16*/

{0x00,0x00,0x10,0x40,0x3F,0xC0,0x00,0x40,0x00,0x00,0x00,0x00},/*"1",17*/

{0x18,0xC0,0x21,0x40,0x22,0x40,0x24,0x40,0x18,0x40,0x00,0x00},/*"2",18*/

{0x10,0x80,0x20,0x40,0x24,0x40,0x24,0x40,0x1B,0x80,0x00,0x00},/*"3",19*/

{0x02,0x00,0x0D,0x00,0x11,0x00,0x3F,0xC0,0x01,0x40,0x00,0x00},/*"4",20*/

{0x3C,0x80,0x24,0x40,0x24,0x40,0x24,0x40,0x23,0x80,0x00,0x00},/*"5",21*/

{0x1F,0x80,0x24,0x40,0x24,0x40,0x34,0x40,0x03,0x80,0x00,0x00},/*"6",22*/

{0x30,0x00,0x20,0x00,0x27,0xC0,0x38,0x00,0x20,0x00,0x00,0x00},/*"7",23*/

{0x1B,0x80,0x24,0x40,0x24,0x40,0x24,0x40,0x1B,0x80,0x00,0x00},/*"8",24*/

{0x1C,0x00,0x22,0xC0,0x22,0x40,0x22,0x40,0x1F,0x80,0x00,0x00},/*"9",25*/

{0x00,0x00,0x00,0x00,0x08,0x40,0x00,0x00,0x00,0x00,0x00,0x00},/*":",26*/

{0x00,0x00,0x00,0x00,0x04,0x60,0x00,0x00,0x00,0x00,0x00,0x00},/*";",27*/

{0x00,0x00,0x04,0x00,0x0A,0x00,0x11,0x00,0x20,0x80,0x40,0x40},/*"<",28*/

{0x09,0x00,0x09,0x00,0x09,0x00,0x09,0x00,0x09,0x00,0x00,0x00},/*"=",29*/

{0x00,0x00,0x40,0x40,0x20,0x80,0x11,0x00,0x0A,0x00,0x04,0x00},/*">",30*/

{0x18,0x00,0x20,0x00,0x23,0x40,0x24,0x00,0x18,0x00,0x00,0x00},/*"?",31*/

{0x1F,0x80,0x20,0x40,0x27,0x40,0x29,0x40,0x1F,0x40,0x00,0x00},/*"@",32*/

{0x00,0x40,0x07,0xC0,0x39,0x00,0x0F,0x00,0x01,0xC0,0x00,0x40},/*"A",33*/

{0x20,0x40,0x3F,0xC0,0x24,0x40,0x24,0x40,0x1B,0x80,0x00,0x00},/*"B",34*/

{0x1F,0x80,0x20,0x40,0x20,0x40,0x20,0x40,0x30,0x80,0x00,0x00},/*"C",35*/

{0x20,0x40,0x3F,0xC0,0x20,0x40,0x20,0x40,0x1F,0x80,0x00,0x00},/*"D",36*/

{0x20,0x40,0x3F,0xC0,0x24,0x40,0x2E,0x40,0x30,0xC0,0x00,0x00},/*"E",37*/

{0x20,0x40,0x3F,0xC0,0x24,0x40,0x2E,0x00,0x30,0x00,0x00,0x00},/*"F",38*/

{0x0F,0x00,0x10,0x80,0x20,0x40,0x22,0x40,0x33,0x80,0x02,0x00},/*"G",39*/

{0x20,0x40,0x3F,0xC0,0x04,0x00,0x04,0x00,0x3F,0xC0,0x20,0x40},/*"H",40*/

{0x20,0x40,0x20,0x40,0x3F,0xC0,0x20,0x40,0x20,0x40,0x00,0x00},/*"I",41*/

{0x00,0x60,0x20,0x20,0x20,0x20,0x3F,0xC0,0x20,0x00,0x20,0x00},/*"J",42*/

{0x20,0x40,0x3F,0xC0,0x24,0x40,0x0B,0x00,0x30,0xC0,0x20,0x40},/*"K",43*/

{0x20,0x40,0x3F,0xC0,0x20,0x40,0x00,0x40,0x00,0x40,0x00,0xC0},/*"L",44*/

{0x3F,0xC0,0x3C,0x00,0x03,0xC0,0x3C,0x00,0x3F,0xC0,0x00,0x00},/*"M",45*/

{0x20,0x40,0x3F,0xC0,0x0C,0x40,0x23,0x00,0x3F,0xC0,0x20,0x00},/*"N",46*/

{0x1F,0x80,0x20,0x40,0x20,0x40,0x20,0x40,0x1F,0x80,0x00,0x00},/*"O",47*/

{0x20,0x40,0x3F,0xC0,0x24,0x40,0x24,0x00,0x18,0x00,0x00,0x00},/*"P",48*/

{0x1F,0x80,0x21,0x40,0x21,0x40,0x20,0xE0,0x1F,0xA0,0x00,0x00},/*"Q",49*/

{0x20,0x40,0x3F,0xC0,0x24,0x40,0x26,0x00,0x19,0xC0,0x00,0x40},/*"R",50*/

{0x18,0xC0,0x24,0x40,0x24,0x40,0x22,0x40,0x31,0x80,0x00,0x00},/*"S",51*/

{0x30,0x00,0x20,0x40,0x3F,0xC0,0x20,0x40,0x30,0x00,0x00,0x00},/*"T",52*/

{0x20,0x00,0x3F,0x80,0x00,0x40,0x00,0x40,0x3F,0x80,0x20,0x00},/*"U",53*/

{0x20,0x00,0x3E,0x00,0x01,0xC0,0x07,0x00,0x38,0x00,0x20,0x00},/*"V",54*/

{0x38,0x00,0x07,0xC0,0x3C,0x00,0x07,0xC0,0x38,0x00,0x00,0x00},/*"W",55*/

{0x20,0x40,0x39,0xC0,0x06,0x00,0x39,0xC0,0x20,0x40,0x00,0x00},/*"X",56*/

{0x20,0x00,0x38,0x40,0x07,0xC0,0x38,0x40,0x20,0x00,0x00,0x00},/*"Y",57*/

{0x30,0x40,0x21,0xC0,0x26,0x40,0x38,0x40,0x20,0xC0,0x00,0x00},/*"Z",58*/

{0x00,0x00,0x00,0x00,0x7F,0xE0,0x40,0x20,0x40,0x20,0x00,0x00},/*"[",59*/

{0x00,0x00,0x70,0x00,0x0C,0x00,0x03,0x80,0x00,0x40,0x00,0x00},/*"\",60*/

{0x00,0x00,0x40,0x20,0x40,0x20,0x7F,0xE0,0x00,0x00,0x00,0x00},/*"]",61*/

{0x00,0x00,0x20,0x00,0x40,0x00,0x20,0x00,0x00,0x00,0x00,0x00},/*"^",62*/

{0x00,0x10,0x00,0x10,0x00,0x10,0x00,0x10,0x00,0x10,0x00,0x10},/*"_",63*/

{0x00,0x00,0x00,0x00,0x40,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"`",64*/

{0x00,0x00,0x02,0x80,0x05,0x40,0x05,0x40,0x03,0xC0,0x00,0x40},/*"a",65*/

{0x20,0x00,0x3F,0xC0,0x04,0x40,0x04,0x40,0x03,0x80,0x00,0x00},/*"b",66*/

{0x00,0x00,0x03,0x80,0x04,0x40,0x04,0x40,0x06,0x40,0x00,0x00},/*"c",67*/

{0x00,0x00,0x03,0x80,0x04,0x40,0x24,0x40,0x3F,0xC0,0x00,0x40},/*"d",68*/

{0x00,0x00,0x03,0x80,0x05,0x40,0x05,0x40,0x03,0x40,0x00,0x00},/*"e",69*/

{0x00,0x00,0x04,0x40,0x1F,0xC0,0x24,0x40,0x24,0x40,0x20,0x00},/*"f",70*/

{0x00,0x00,0x02,0xE0,0x05,0x50,0x05,0x50,0x06,0x50,0x04,0x20},/*"g",71*/

{0x20,0x40,0x3F,0xC0,0x04,0x40,0x04,0x00,0x03,0xC0,0x00,0x40},/*"h",72*/

{0x00,0x00,0x04,0x40,0x27,0xC0,0x00,0x40,0x00,0x00,0x00,0x00},/*"i",73*/

{0x00,0x10,0x00,0x10,0x04,0x10,0x27,0xE0,0x00,0x00,0x00,0x00},/*"j",74*/

{0x20,0x40,0x3F,0xC0,0x01,0x40,0x07,0x00,0x04,0xC0,0x04,0x40},/*"k",75*/

{0x20,0x40,0x20,0x40,0x3F,0xC0,0x00,0x40,0x00,0x40,0x00,0x00},/*"l",76*/

{0x07,0xC0,0x04,0x00,0x07,0xC0,0x04,0x00,0x03,0xC0,0x00,0x00},/*"m",77*/

{0x04,0x40,0x07,0xC0,0x04,0x40,0x04,0x00,0x03,0xC0,0x00,0x40},/*"n",78*/

{0x00,0x00,0x03,0x80,0x04,0x40,0x04,0x40,0x03,0x80,0x00,0x00},/*"o",79*/

{0x04,0x10,0x07,0xF0,0x04,0x50,0x04,0x40,0x03,0x80,0x00,0x00},/*"p",80*/

{0x00,0x00,0x03,0x80,0x04,0x40,0x04,0x50,0x07,0xF0,0x00,0x10},/*"q",81*/

{0x04,0x40,0x07,0xC0,0x02,0x40,0x04,0x00,0x04,0x00,0x00,0x00},/*"r",82*/

{0x00,0x00,0x06,0x40,0x05,0x40,0x05,0x40,0x04,0xC0,0x00,0x00},/*"s",83*/

{0x00,0x00,0x04,0x00,0x1F,0x80,0x04,0x40,0x00,0x40,0x00,0x00},/*"t",84*/

{0x04,0x00,0x07,0x80,0x00,0x40,0x04,0x40,0x07,0xC0,0x00,0x40},/*"u",85*/

{0x04,0x00,0x07,0x00,0x04,0xC0,0x01,0x80,0x06,0x00,0x04,0x00},/*"v",86*/

{0x06,0x00,0x01,0xC0,0x07,0x00,0x01,0xC0,0x06,0x00,0x00,0x00},/*"w",87*/

{0x04,0x40,0x06,0xC0,0x01,0x00,0x06,0xC0,0x04,0x40,0x00,0x00},/*"x",88*/

{0x04,0x10,0x07,0x10,0x04,0xE0,0x01,0x80,0x06,0x00,0x04,0x00},/*"y",89*/

{0x00,0x00,0x04,0x40,0x05,0xC0,0x06,0x40,0x04,0x40,0x00,0x00},/*"z",90*/

{0x00,0x00,0x00,0x00,0x04,0x00,0x7B,0xE0,0x40,0x20,0x00,0x00},/*"{",91*/

{0x00,0x00,0x00,0x00,0x00,0x00,0xFF,0xF0,0x00,0x00,0x00,0x00},/*"|",92*/

{0x00,0x00,0x40,0x20,0x7B,0xE0,0x04,0x00,0x00,0x00,0x00,0x00},/*"}",93*/

{0x40,0x00,0x80,0x00,0x40,0x00,0x20,0x00,0x20,0x00,0x40,0x00},/*"~",94*/

};

const unsigned char oled_asc2_1608[95][16]={

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*" ",0*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x1F,0xCC,0x00,0x0C,0x00,0x00,0x00,0x00,0x00,0x00},/*"!",1*/

{0x00,0x00,0x08,0x00,0x30,0x00,0x60,0x00,0x08,0x00,0x30,0x00,0x60,0x00,0x00,0x00},/*""",2*/

{0x02,0x20,0x03,0xFC,0x1E,0x20,0x02,0x20,0x03,0xFC,0x1E,0x20,0x02,0x20,0x00,0x00},/*"#",3*/

{0x00,0x00,0x0E,0x18,0x11,0x04,0x3F,0xFF,0x10,0x84,0x0C,0x78,0x00,0x00,0x00,0x00},/*"$",4*/

{0x0F,0x00,0x10,0x84,0x0F,0x38,0x00,0xC0,0x07,0x78,0x18,0x84,0x00,0x78,0x00,0x00},/*"%",5*/

{0x00,0x78,0x0F,0x84,0x10,0xC4,0x11,0x24,0x0E,0x98,0x00,0xE4,0x00,0x84,0x00,0x08},/*"&",6*/

{0x08,0x00,0x68,0x00,0x70,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"'",7*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x07,0xE0,0x18,0x18,0x20,0x04,0x40,0x02,0x00,0x00},/*"(",8*/

{0x00,0x00,0x40,0x02,0x20,0x04,0x18,0x18,0x07,0xE0,0x00,0x00,0x00,0x00,0x00,0x00},/*")",9*/

{0x02,0x40,0x02,0x40,0x01,0x80,0x0F,0xF0,0x01,0x80,0x02,0x40,0x02,0x40,0x00,0x00},/*"*",10*/

{0x00,0x80,0x00,0x80,0x00,0x80,0x0F,0xF8,0x00,0x80,0x00,0x80,0x00,0x80,0x00,0x00},/*"+",11*/

{0x00,0x01,0x00,0x0D,0x00,0x0E,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*",",12*/

{0x00,0x00,0x00,0x80,0x00,0x80,0x00,0x80,0x00,0x80,0x00,0x80,0x00,0x80,0x00,0x80},/*"-",13*/

{0x00,0x00,0x00,0x0C,0x00,0x0C,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*".",14*/

{0x00,0x00,0x00,0x06,0x00,0x18,0x00,0x60,0x01,0x80,0x06,0x00,0x18,0x00,0x20,0x00},/*"/",15*/

{0x00,0x00,0x07,0xF0,0x08,0x08,0x10,0x04,0x10,0x04,0x08,0x08,0x07,0xF0,0x00,0x00},/*"0",16*/

{0x00,0x00,0x08,0x04,0x08,0x04,0x1F,0xFC,0x00,0x04,0x00,0x04,0x00,0x00,0x00,0x00},/*"1",17*/

{0x00,0x00,0x0E,0x0C,0x10,0x14,0x10,0x24,0x10,0x44,0x11,0x84,0x0E,0x0C,0x00,0x00},/*"2",18*/

{0x00,0x00,0x0C,0x18,0x10,0x04,0x11,0x04,0x11,0x04,0x12,0x88,0x0C,0x70,0x00,0x00},/*"3",19*/

{0x00,0x00,0x00,0xE0,0x03,0x20,0x04,0x24,0x08,0x24,0x1F,0xFC,0x00,0x24,0x00,0x00},/*"4",20*/

{0x00,0x00,0x1F,0x98,0x10,0x84,0x11,0x04,0x11,0x04,0x10,0x88,0x10,0x70,0x00,0x00},/*"5",21*/

{0x00,0x00,0x07,0xF0,0x08,0x88,0x11,0x04,0x11,0x04,0x18,0x88,0x00,0x70,0x00,0x00},/*"6",22*/

{0x00,0x00,0x1C,0x00,0x10,0x00,0x10,0xFC,0x13,0x00,0x1C,0x00,0x10,0x00,0x00,0x00},/*"7",23*/

{0x00,0x00,0x0E,0x38,0x11,0x44,0x10,0x84,0x10,0x84,0x11,0x44,0x0E,0x38,0x00,0x00},/*"8",24*/

{0x00,0x00,0x07,0x00,0x08,0x8C,0x10,0x44,0x10,0x44,0x08,0x88,0x07,0xF0,0x00,0x00},/*"9",25*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x03,0x0C,0x03,0x0C,0x00,0x00,0x00,0x00,0x00,0x00},/*":",26*/

{0x00,0x00,0x00,0x00,0x00,0x01,0x01,0x06,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*";",27*/

{0x00,0x00,0x00,0x80,0x01,0x40,0x02,0x20,0x04,0x10,0x08,0x08,0x10,0x04,0x00,0x00},/*"<",28*/

{0x02,0x20,0x02,0x20,0x02,0x20,0x02,0x20,0x02,0x20,0x02,0x20,0x02,0x20,0x00,0x00},/*"=",29*/

{0x00,0x00,0x10,0x04,0x08,0x08,0x04,0x10,0x02,0x20,0x01,0x40,0x00,0x80,0x00,0x00},/*">",30*/

{0x00,0x00,0x0E,0x00,0x12,0x00,0x10,0x0C,0x10,0x6C,0x10,0x80,0x0F,0x00,0x00,0x00},/*"?",31*/

{0x03,0xE0,0x0C,0x18,0x13,0xE4,0x14,0x24,0x17,0xC4,0x08,0x28,0x07,0xD0,0x00,0x00},/*"@",32*/

{0x00,0x04,0x00,0x3C,0x03,0xC4,0x1C,0x40,0x07,0x40,0x00,0xE4,0x00,0x1C,0x00,0x04},/*"A",33*/

{0x10,0x04,0x1F,0xFC,0x11,0x04,0x11,0x04,0x11,0x04,0x0E,0x88,0x00,0x70,0x00,0x00},/*"B",34*/

{0x03,0xE0,0x0C,0x18,0x10,0x04,0x10,0x04,0x10,0x04,0x10,0x08,0x1C,0x10,0x00,0x00},/*"C",35*/

{0x10,0x04,0x1F,0xFC,0x10,0x04,0x10,0x04,0x10,0x04,0x08,0x08,0x07,0xF0,0x00,0x00},/*"D",36*/

{0x10,0x04,0x1F,0xFC,0x11,0x04,0x11,0x04,0x17,0xC4,0x10,0x04,0x08,0x18,0x00,0x00},/*"E",37*/

{0x10,0x04,0x1F,0xFC,0x11,0x04,0x11,0x00,0x17,0xC0,0x10,0x00,0x08,0x00,0x00,0x00},/*"F",38*/

{0x03,0xE0,0x0C,0x18,0x10,0x04,0x10,0x04,0x10,0x44,0x1C,0x78,0x00,0x40,0x00,0x00},/*"G",39*/

{0x10,0x04,0x1F,0xFC,0x10,0x84,0x00,0x80,0x00,0x80,0x10,0x84,0x1F,0xFC,0x10,0x04},/*"H",40*/

{0x00,0x00,0x10,0x04,0x10,0x04,0x1F,0xFC,0x10,0x04,0x10,0x04,0x00,0x00,0x00,0x00},/*"I",41*/

{0x00,0x03,0x00,0x01,0x10,0x01,0x10,0x01,0x1F,0xFE,0x10,0x00,0x10,0x00,0x00,0x00},/*"J",42*/

{0x10,0x04,0x1F,0xFC,0x11,0x04,0x03,0x80,0x14,0x64,0x18,0x1C,0x10,0x04,0x00,0x00},/*"K",43*/

{0x10,0x04,0x1F,0xFC,0x10,0x04,0x00,0x04,0x00,0x04,0x00,0x04,0x00,0x0C,0x00,0x00},/*"L",44*/

{0x10,0x04,0x1F,0xFC,0x1F,0x00,0x00,0xFC,0x1F,0x00,0x1F,0xFC,0x10,0x04,0x00,0x00},/*"M",45*/

{0x10,0x04,0x1F,0xFC,0x0C,0x04,0x03,0x00,0x00,0xE0,0x10,0x18,0x1F,0xFC,0x10,0x00},/*"N",46*/

{0x07,0xF0,0x08,0x08,0x10,0x04,0x10,0x04,0x10,0x04,0x08,0x08,0x07,0xF0,0x00,0x00},/*"O",47*/

{0x10,0x04,0x1F,0xFC,0x10,0x84,0x10,0x80,0x10,0x80,0x10,0x80,0x0F,0x00,0x00,0x00},/*"P",48*/

{0x07,0xF0,0x08,0x18,0x10,0x24,0x10,0x24,0x10,0x1C,0x08,0x0A,0x07,0xF2,0x00,0x00},/*"Q",49*/

{0x10,0x04,0x1F,0xFC,0x11,0x04,0x11,0x00,0x11,0xC0,0x11,0x30,0x0E,0x0C,0x00,0x04},/*"R",50*/

{0x00,0x00,0x0E,0x1C,0x11,0x04,0x10,0x84,0x10,0x84,0x10,0x44,0x1C,0x38,0x00,0x00},/*"S",51*/

{0x18,0x00,0x10,0x00,0x10,0x04,0x1F,0xFC,0x10,0x04,0x10,0x00,0x18,0x00,0x00,0x00},/*"T",52*/

{0x10,0x00,0x1F,0xF8,0x10,0x04,0x00,0x04,0x00,0x04,0x10,0x04,0x1F,0xF8,0x10,0x00},/*"U",53*/

{0x10,0x00,0x1E,0x00,0x11,0xE0,0x00,0x1C,0x00,0x70,0x13,0x80,0x1C,0x00,0x10,0x00},/*"V",54*/

{0x1F,0xC0,0x10,0x3C,0x00,0xE0,0x1F,0x00,0x00,0xE0,0x10,0x3C,0x1F,0xC0,0x00,0x00},/*"W",55*/

{0x10,0x04,0x18,0x0C,0x16,0x34,0x01,0xC0,0x01,0xC0,0x16,0x34,0x18,0x0C,0x10,0x04},/*"X",56*/

{0x10,0x00,0x1C,0x00,0x13,0x04,0x00,0xFC,0x13,0x04,0x1C,0x00,0x10,0x00,0x00,0x00},/*"Y",57*/

{0x08,0x04,0x10,0x1C,0x10,0x64,0x10,0x84,0x13,0x04,0x1C,0x04,0x10,0x18,0x00,0x00},/*"Z",58*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x7F,0xFE,0x40,0x02,0x40,0x02,0x40,0x02,0x00,0x00},/*"[",59*/

{0x00,0x00,0x30,0x00,0x0C,0x00,0x03,0x80,0x00,0x60,0x00,0x1C,0x00,0x03,0x00,0x00},/*"\",60*/

{0x00,0x00,0x40,0x02,0x40,0x02,0x40,0x02,0x7F,0xFE,0x00,0x00,0x00,0x00,0x00,0x00},/*"]",61*/

{0x00,0x00,0x00,0x00,0x20,0x00,0x40,0x00,0x40,0x00,0x40,0x00,0x20,0x00,0x00,0x00},/*"^",62*/

{0x00,0x01,0x00,0x01,0x00,0x01,0x00,0x01,0x00,0x01,0x00,0x01,0x00,0x01,0x00,0x01},/*"_",63*/

{0x00,0x00,0x40,0x00,0x40,0x00,0x20,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"`",64*/

{0x00,0x00,0x00,0x98,0x01,0x24,0x01,0x44,0x01,0x44,0x01,0x44,0x00,0xFC,0x00,0x04},/*"a",65*/

{0x10,0x00,0x1F,0xFC,0x00,0x88,0x01,0x04,0x01,0x04,0x00,0x88,0x00,0x70,0x00,0x00},/*"b",66*/

{0x00,0x00,0x00,0x70,0x00,0x88,0x01,0x04,0x01,0x04,0x01,0x04,0x00,0x88,0x00,0x00},/*"c",67*/

{0x00,0x00,0x00,0x70,0x00,0x88,0x01,0x04,0x01,0x04,0x11,0x08,0x1F,0xFC,0x00,0x04},/*"d",68*/

{0x00,0x00,0x00,0xF8,0x01,0x44,0x01,0x44,0x01,0x44,0x01,0x44,0x00,0xC8,0x00,0x00},/*"e",69*/

{0x00,0x00,0x01,0x04,0x01,0x04,0x0F,0xFC,0x11,0x04,0x11,0x04,0x11,0x00,0x18,0x00},/*"f",70*/

{0x00,0x00,0x00,0xD6,0x01,0x29,0x01,0x29,0x01,0x29,0x01,0xC9,0x01,0x06,0x00,0x00},/*"g",71*/

{0x10,0x04,0x1F,0xFC,0x00,0x84,0x01,0x00,0x01,0x00,0x01,0x04,0x00,0xFC,0x00,0x04},/*"h",72*/

{0x00,0x00,0x01,0x04,0x19,0x04,0x19,0xFC,0x00,0x04,0x00,0x04,0x00,0x00,0x00,0x00},/*"i",73*/

{0x00,0x00,0x00,0x03,0x00,0x01,0x01,0x01,0x19,0x01,0x19,0xFE,0x00,0x00,0x00,0x00},/*"j",74*/

{0x10,0x04,0x1F,0xFC,0x00,0x24,0x00,0x40,0x01,0xB4,0x01,0x0C,0x01,0x04,0x00,0x00},/*"k",75*/

{0x00,0x00,0x10,0x04,0x10,0x04,0x1F,0xFC,0x00,0x04,0x00,0x04,0x00,0x00,0x00,0x00},/*"l",76*/

{0x01,0x04,0x01,0xFC,0x01,0x04,0x01,0x00,0x01,0xFC,0x01,0x04,0x01,0x00,0x00,0xFC},/*"m",77*/

{0x01,0x04,0x01,0xFC,0x00,0x84,0x01,0x00,0x01,0x00,0x01,0x04,0x00,0xFC,0x00,0x04},/*"n",78*/

{0x00,0x00,0x00,0xF8,0x01,0x04,0x01,0x04,0x01,0x04,0x01,0x04,0x00,0xF8,0x00,0x00},/*"o",79*/

{0x01,0x01,0x01,0xFF,0x00,0x85,0x01,0x04,0x01,0x04,0x00,0x88,0x00,0x70,0x00,0x00},/*"p",80*/

{0x00,0x00,0x00,0x70,0x00,0x88,0x01,0x04,0x01,0x04,0x01,0x05,0x01,0xFF,0x00,0x01},/*"q",81*/

{0x01,0x04,0x01,0x04,0x01,0xFC,0x00,0x84,0x01,0x04,0x01,0x00,0x01,0x80,0x00,0x00},/*"r",82*/

{0x00,0x00,0x00,0xCC,0x01,0x24,0x01,0x24,0x01,0x24,0x01,0x24,0x01,0x98,0x00,0x00},/*"s",83*/

{0x00,0x00,0x01,0x00,0x01,0x00,0x07,0xF8,0x01,0x04,0x01,0x04,0x00,0x00,0x00,0x00},/*"t",84*/

{0x01,0x00,0x01,0xF8,0x00,0x04,0x00,0x04,0x00,0x04,0x01,0x08,0x01,0xFC,0x00,0x04},/*"u",85*/

{0x01,0x00,0x01,0x80,0x01,0x70,0x00,0x0C,0x00,0x10,0x01,0x60,0x01,0x80,0x01,0x00},/*"v",86*/

{0x01,0xF0,0x01,0x0C,0x00,0x30,0x01,0xC0,0x00,0x30,0x01,0x0C,0x01,0xF0,0x01,0x00},/*"w",87*/

{0x00,0x00,0x01,0x04,0x01,0x8C,0x00,0x74,0x01,0x70,0x01,0x8C,0x01,0x04,0x00,0x00},/*"x",88*/

{0x01,0x01,0x01,0x81,0x01,0x71,0x00,0x0E,0x00,0x18,0x01,0x60,0x01,0x80,0x01,0x00},/*"y",89*/

{0x00,0x00,0x01,0x84,0x01,0x0C,0x01,0x34,0x01,0x44,0x01,0x84,0x01,0x0C,0x00,0x00},/*"z",90*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x01,0x00,0x3E,0xFC,0x40,0x02,0x40,0x02},/*"{",91*/

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0xFF,0xFF,0x00,0x00,0x00,0x00,0x00,0x00},/*"|",92*/

{0x00,0x00,0x40,0x02,0x40,0x02,0x3E,0xFC,0x01,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"}",93*/

{0x00,0x00,0x60,0x00,0x80,0x00,0x80,0x00,0x40,0x00,0x40,0x00,0x20,0x00,0x20,0x00},/*"~",94*/

};

const unsigned char oled_CH_1616[][12]={

/////////////////////////////////////////////////////////////

{0x00,0x00,0x4F,0xF0,0x28,0x00,0x0B,0xC0,0x0A,0x40,0xFA,0x40},

{0x0A,0x40,0x0B,0xC0,0x28,0x10,0x4F,0xF0,0x00,0x00,0x00,0x00},/*"尚",0*/

{0x42,0x00,0x4F,0xE0,0x74,0x40,0x47,0xE0,0x00,0x10,0x24,0x90},

{0x24,0x90,0xFD,0xF0,0x24,0x90,0x24,0x90,0x04,0x10,0x00,0x00},/*"硅",1*/

{0x02,0x00,0x22,0x00,0x45,0xF0,0x89,0x20,0x11,0x20,0x21,0x20},

{0x11,0x20,0x89,0x20,0x45,0xF0,0x22,0x00,0x02,0x00,0x00,0x00},/*"谷",2*/

{0x04,0x00,0x6F,0xF0,0x24,0x90,0x24,0x90,0x2F,0xF0,0xE4,0x00},

{0x22,0x10,0x3C,0x60,0x25,0x80,0x64,0x60,0x06,0x10,0x00,0x00},/*"嵌",3*/

{0x00,0x10,0x00,0x20,0x80,0x40,0x81,0x80,0x4E,0x00,0x30,0x00},

{0x0E,0x00,0x01,0x80,0x00,0x40,0x00,0x20,0x00,0x10,0x00,0x00},/*"入",4*/

{0x10,0x10,0x12,0x10,0x12,0x10,0x13,0xE0,0x12,0x20,0x12,0x20},

{0x10,0x20,0xFF,0x00,0x10,0xC0,0x90,0x20,0x50,0x70,0x00,0x00},/*"式",5*/

/////////////////////////////////////////////////////////////

//信号图标

{0x00,0x00,0x00,0x00,0x00,0x00,0x10,0x80,0x09,0x00,0x06,0x00},

{0x06,0x00,0x09,0x00,0x10,0x80,0x00,0x00,0x00,0x00,0x00,0x00},/*"x   ",0*/

{0x00,0x00,0x00,0x30,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"一竖",1*/

{0x00,0x00,0x00,0x30,0x00,0x00,0x00,0xF0,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"两竖",2*/

{0x00,0x00,0x00,0x30,0x00,0x00,0x00,0xF0,0x00,0x00,0x03,0xF0},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"三竖",3*/

{0x00,0x00,0x00,0x30,0x00,0x00,0x00,0xF0,0x00,0x00,0x03,0xF0},

{0x00,0x00,0x0F,0xF0,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*"四竖",4*/

{0x00,0x00,0x00,0x30,0x00,0x00,0x00,0xF0,0x00,0x00,0x03,0xF0},

{0x00,0x00,0x0F,0xF0,0x00,0x00,0x3F,0xF0,0x00,0x00,0x00,0x00},/*"五竖",5*/

/////////////////////////////////////////////////////////////

//进度条

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//6

{0x7F,0xE0,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//7

{0x7F,0xE0,0x7F,0xE0,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//8

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x00,0x00,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//9

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//10

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//11

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//12

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x7F,0xE0,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//13

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x7F,0xE0,0x7F,0xE0,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},//14

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x00,0x00,0x00,0x00,0x00,0x00},//15

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x00,0x00,0x00,0x00},//16

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x00,0x00},//17

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},

{0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0,0x7F,0xE0},//18

/////////////////////////////////////////////////////////////

//电灯图标

{0x00,0x00,0x00,0x00,0x4C,0x00,0x2A,0x00,0x09,0xE0,0x28,0x20},

{0x28,0x20,0x09,0xE0,0x2A,0x00,0x4C,0x00,0x00,0x00,0x00,0x00},/*"开",19*/

{0x00,0x00,0x40,0x20,0x26,0x40,0x15,0x80,0x0D,0xE0,0x16,0x20},

{0x16,0x20,0x0D,0xE0,0x15,0x80,0x26,0x40,0x40,0x20,0x00,0x00},/*"关",20*/

};

const unsigned char oled_CH15[][12]={

{0x21,0x00,0xA2,0x00,0x6F,0xF0,0x34,0x00,0x02,0x10,0x7F,0x20},

{0x40,0xC0,0x5F,0x00,0x40,0xE0,0x7F,0x10,0x00,0x30,0x00,0x00},/*"?",0*/

{0x38,0x10,0xA0,0x10,0x6F,0xA0,0x28,0x20,0xA8,0x40,0x6B,0x80},

{0x28,0x60,0x28,0x10,0x6F,0x90,0xA0,0x10,0x38,0x30,0x00,0x00},/*"?",1*/

{0x34,0x10,0x24,0x20,0x25,0xC0,0x24,0x20,0xA4,0x10,0x67,0xF0},

{0x24,0x90,0x24,0x90,0x24,0x90,0x24,0x90,0x34,0x10,0x00,0x00},/*"?",2*/

{0x08,0x00,0x3F,0xF0,0xC0,0x00,0x10,0x10,0x16,0x10,0x11,0xD0},

{0x90,0x10,0x50,0x30,0x10,0xD0,0x17,0x10,0x10,0x10,0x00,0x00},/*"?",3*/

{0x11,0x00,0x16,0x00,0xFF,0xF0,0x12,0x00,0x40,0x50,0x5F,0x50},

{0xF5,0x60,0x55,0xC0,0xF5,0x60,0x5F,0x50,0x40,0x50,0x00,0x00},/*"?",4*/

{0x10,0x40,0xFF,0xC0,0x10,0x80,0x02,0x10,0x22,0x20,0x22,0xC0},

{0xFF,0x00,0x22,0xC0,0x22,0x20,0x3E,0x10,0x02,0x10,0x00,0x00},/*"?",5*/

{0x00,0x00,0x00,0x00,0x0C,0xC0,0x0C,0xC0,0x00,0x00,0x00,0x00},

{0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00},/*":",6*/

};

应用层

App_Display.h

#ifndef __APP_DISPLAY_H

#define __APP_DISPLAY_H

#include "Common_Debug.h"

#include "oled.h"

#include "oledfont.h"

#include "Inf_Si24R1.h"

#include "Inf_JoyStickKey.h"

#define Line1_Begin 29

#define Line2_Begin 5

#define Line3_Begin 5

#define Line4_Begin 30

#define Line5_Begin 2

#define X_Begin 0

#define Y_Begin 51

#define Z_Begin 103

#define Line1_Begin1 0

#define Line2_Begin1 0

#define Line3_Begin1 40

#define Line4_Begin1 0

#define Line5_Begin1 0

#define Y0 0

#define Y1 14

#define Y2 Y1 + 12

#define Y3 Y2 + 12

#define Y4 Y3 + 12

#define Y5 Y4 + 12

typedef struct

{

uint8_t  hardware_type;

uint8_t  oled_delay;

uint8_t  windows;

uint8_t  low_power;

uint8_t  NRF_Err;

uint8_t  Rc_num;

uint8_t  Connect_Succeed;

uint16_t Battery_Rc;

uint16_t Battery_Fly;

uint16_t test_flag, set_flag;

int      X, Y, Z, H;

} Show_Struct;

extern Show_Struct Show;

extern uint16_t    set_temp;

extern uint16_t    test_temp;

extern uint8_t     send_flag;

void App_Display_Start(void);

void App_Display_Show(void);

#endif

App_Display.c

#include "App_Display.h"

void App_Display_ShowProgressBar(uint8_t temp,

uint8_t chr_star,

uint8_t chr_default,

uint8_t x,

uint8_t y,

uint8_t size,

uint8_t mode);

void App_Display_Start(void)

{

OLED_Init();

}

void App_Display_Show(void)

{

uint8_t temp;

///////////////////////////////第一行///////////////////////////////////

OLED_Show_CH(Line1_Begin + 00, Y0, 0, 12, 1);

OLED_Show_CH(Line1_Begin + 12, Y0, 1, 12, 1);

OLED_Show_CH(Line1_Begin + 24, Y0, 2, 12, 1);

OLED_Show_CH(Line1_Begin + 36, Y0, 3, 12, 1);

OLED_Show_CH(Line1_Begin + 48, Y0, 4, 12, 1);

OLED_Show_CH(Line1_Begin + 60, Y0, 5, 12, 1);

OLED_ShowNumber(2, Y0, CH, 3, 12);   // 显示无线信道

///////////////////////////////第二行///////////////////////////////////

///////////////////////////////第三、四行/////////////////////////////////

// 显示遥控数据

OLED_ShowString(Line3_Begin + 00, Y2, "THR:", 12, 1);

temp = joyStick.THR / 41;

App_Display_ShowProgressBar(temp, 12, 24, Line4_Begin + 6, Y2, 12, 1);

temp = (joyStick.THR - 500) / 41;

App_Display_ShowProgressBar(temp, 12, 12, Line4_Begin + 18, Y2, 12, 1);

OLED_ShowString(Line3_Begin + 64, Y2, "ROL:", 12, 1);

temp = joyStick.ROL / 41;

App_Display_ShowProgressBar(temp, 12, 24, Line4_Begin + 70, Y2, 12, 1);

temp = (joyStick.ROL - 500) / 41;

App_Display_ShowProgressBar(temp, 12, 12, Line4_Begin + 82, Y2, 12, 1);

OLED_ShowString(Line3_Begin + 00, Y3, "YAW:", 12, 1);

temp = joyStick.YAW / 41;

App_Display_ShowProgressBar(temp, 12, 24, Line4_Begin + 6, Y3, 12, 1);

temp = (joyStick.YAW - 500) / 41;

App_Display_ShowProgressBar(temp, 12, 12, Line4_Begin + 18, Y3, 12, 1);

OLED_ShowString(Line3_Begin + 64, Y3, "PIT:", 12, 1);

temp = joyStick.PIT / 41;

App_Display_ShowProgressBar(temp, 12, 24, Line4_Begin + 70, Y3, 12, 1);

temp = (joyStick.PIT - 500) / 41;

App_Display_ShowProgressBar(temp, 12, 12, Line4_Begin + 82, Y3, 12, 1);

OLED_Refresh_Gram();   // 开始显示

}

// 进度条显示函数

void App_Display_ShowProgressBar(uint8_t temp,

uint8_t chr_star,

uint8_t chr_default,

uint8_t x,

uint8_t y,

uint8_t size,

uint8_t mode)

{

switch(temp)

{

case 0: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 1: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 2: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 3: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 4: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 5: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 6: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 7: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 8: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 9: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 10: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 11: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

case 12: OLED_Show_CH(x, y, chr_star + temp, size, size); break;

default: OLED_Show_CH(x, y, chr_default, size, size); break;

}

}

任务调度

/* 4. 显示任务 */

void         displayTask(void *arg);

TaskHandle_t displayHandle;

#define DISPLAY_PRIORITY 8

#define DISPLAY_EXECUTE_CYCLE 10

/* 4. 显示任务 */

void displayTask(void *arg)

{

debug_printfln("显示任务: 开始调度");

TickType_t pxPreviousWakeTime = xTaskGetTickCount();

while(1)

{

App_Display_Show();

xTaskDelayUntil(&pxPreviousWakeTime, JOYSTICK_EXECUTE_CYCLE);

}

}